POS scale/ barcode scale RS232 communication protocol and setting（2015-10-26）

POS scale setting：

Switch on the scale, after entering the weighing state, press number key【1】+【CHG】（24keys）or number key【1】+【total】（16key）at the same time to enter RS232 setting state.

- Price window display“CLOSE”or “SER”，press【tare】key to select，“CLOSE”means COM is closed, SER means COM is opened. Press 【A.C./SET】key to confirm and enter next step

- Price window display “B ****”，press【tare】key to select the baud rate, the option baud rate is 1200，2400，4800，9600. Press【A.C./SET】key to confirm and enter next step

- Price window display “P   **”，press【tare】key to select communication protocol, optional from 1－12. Press【A.C./SET】key to confirm and enter next step

- If select the protocol 1， then start to set the machine address, otherwise directly finish the RS232 setting. Price window display “A  ***”, press number key to enter machine address（0-254），press【A.C./SET】key to confirm and finish the RS232 setting

Barcode scale setting:

Change the parameter spec57 to the corresponding protocol number

Protocol list

| Protocol No. | Protocol name | Sending method | explaination |
| --- | --- | --- | --- |
| 1 | Protocol A | Continuous transmission | Include address, unit price, price |
| 2 | Protocol B | Continuous transmission | Only weight |
| 3 | Protocol C | Continuous transmission | Only weight |
| 4 | Protocol D | Continuous transmission | Only weight |
| 5 | Protocol D | Stable transmission | Only weight |
| 6 | Protocol E | Continuous transmission | Only weight |
| 7 | Protocol E | Stable transmission | Only weight |
| 8 | Protocol F | Continuous transmission | Include unit price, price |
| 9 | Protocol F | Stable transmission | Include unit price, price |
| 10 | Protocol G | Continuous transmission | Only weight |
| 11 | Protocol G | Stable transmission | Only weight |
| 12 | Protocol H | Reply transmission | Only weight (compatible Dahua protocol) |

Communication protocol：

RS232 protocol： 8 data bit，1 stop bit，no check bit

Data formats：

Protocol A（total 14 bytes）

No. 1：0xFFstart bit

No. 2：D0~D2 — Decimal point position for weight window(1-4，1 means no decimal point, 4 means 3 decimal point)

D3 —  1 means weight is negative，0 means weight is positive

D4 —  1 means weight is stable，0 means weight is unstable

D5 —  1 means weight is overflow，0 means weight is not overflow

D6 —  1 means price accumulate or look over the total price, the sending weight number is 0, 0 means the sending data is current weight

D7 —  1 means the unit is not Kg, 0 means the unit is Kg

No. 3：D0~D7 — BCD1（LSB for net weight）

No. 4：D0~D7 — BCD2（MSB for net weight）

No. 5：D0~D7 — BCD3（HSB for net weight）

No. 6：D0~D2 — unit price decimal point position（1－4，1 means no decimal point,4 means 3 decimal point）

D3 — 1 means price accumulate or look over the total price, the sending unit price number is the accumulate times, the unit price decimal point position be 1，0 means the sending data is current unit price

D4~D7 — NC

No. 7：D0~D7 — BCD4（LSB for unit price or accumulate times）

No. 8：D0~D7 — BCD5（MSB for unit price or accumulate times）

No. 9：D0~D7 — BCD6（HSB for unit price or accumulate times）

No. 10：D0~D2 — decimal point position for price（1－4，1 means no decimal point,4 means 3 decimal point）

D3 — 1 means price accumulate or look over the total price, the sending price number is accumulate price, 0 means the sending data is current price

D4 — 1 means the price overflow, 0 means the price is not overflow；

D5 — 1 means the weight is negative, the present price is invalidation data；

D6~D7 —  NC

No. 11：D0~D7 — BCD7（LSB for price）

No. 12：D0~D7 — BCD8（MSB for price）

No. 13：D0~D7 — BCD9（HSB for price）

No. 14：D0~D7 — ADDRESS：user defined machine address, range is 0-254

For example：

Net weight 2.365；unit price 5.62；price 13.29，present weight stable，machine address 200，Send data as follows (hexadecimal)：

FF 14 65 23 00 03 62 05 00 03 29 13 00 C8

Protocol B（total 6 bytes）

No. 1：0xFF start bit

No. 2：D0~D2 — decimal point position for price（1－4，1 means no decimal point,4 means 3 decimal point）

D3~D4 —  hold 0

D5 — 1 means weight is negative，0 means weight is positive

D6 — 1 means weight is stable，0 means weight is unstable

D7 — 1 means weight is overflow，0 means weight is not overflow

No. 3：D0~D7 — BCD1（LSB for net weight）

No. 4：D0~D7 — BCD2（MSB for net weight）

No. 5：D0~D7 — BCD3（HSB for net weight）

No. 6：D0~D7 — 1 means the unit is not Kg, 0 means the unit is Kg；

For example：

Net weight 2.365，present weight stable，send data as follows (hexadecimal)：

FF 44 65 23 00 00

Protocol C（total 18 bytes）

No.1－2（ASCII）：WT

No. 3－4（ASCII）：“ST"-stable, "US"-unstable, "OL"-over load

No. 5（ASCII）：weight is positive ＋，weight is negative －

No. 6－12（ASCII）：weight，for example 1.235；Insufficient part fill with space

No. 13－16（ASCII）：weight unit，Insufficient part fill with space，for example  kg

No. 17（hexadecimal）： 0x0D

No. 18（hexadecimal）： 0x0A

For example：

Net weight 2.365，present weight stable，send data as follows：

| No. | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ASCII | W | T | S | T | + |  |  | 2 | . | 3 | 6 | 5 |  |  | k | g |  |  |
| hex | 57 | 54 | 53 | 54 | 2B | 20 | 20 | 32 | 2E | 33 | 36 | 35 | 20 | 20 | 6B | 67 | 0D | 0A |

Protocol D（total 8 bytes）

No. 1（ASCII）：= start bit

No. 2－8（ASCII）：Send 6-bit weight data in the order of low first and then high, and finally send symbol bit, where "-" represents negative number, space represents positive number, and less than 8 data are filled with space

For example:

Net weight 2.365, send data as follows：

| No. | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ASCII | = | 5 | 6 | 3 | . | 2 |  |  |
| hex | 3D | 35 | 36 | 33 | 2E | 32 | 20 | 20 |

Protocol E（total 12 bytes）

No.1（hex）：0x02 start bit

No.2（ASCII）：The weight is positive transmit +,  the weight is negative transmit -

No.3－8（ASCII）：weight，the deficiency is made up with 0

No.9（ASCII）：Number of decimal places（0～4）

No.10（ASCII）：XOR check, high four bits（0－9，A－F）

No.11（ASCII）：XOR check, low four bits（0－9，A－F）

No.12（hex）：0x03

XOR＝2⊕3⊕……8⊕9

For example：

Net weight 2.365，send data as follows：

| No. | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ASCII |  | + | 0 | 0 | 2 | 3 | 6 | 5 | 3 | 1 | A |  |
| hex | 02 | 2B | 30 | 30 | 32 | 33 | 36 | 35 | 33 | 31 | 41 | 03 |

XOR＝2B⊕30⊕30⊕32⊕33⊕36⊕35⊕33＝1A

Protocol F（total 22 bytes）

No. 1（hex）：0x0A

No. 2（hex）：0x0D

No.3－7（ASCII）：Weight data, the insufficient part is supplemented with space, excluding the decimal point

No.8，9（ASCII）：Space

No.10－14（ASCII）：Unit price data, the insufficient part shall be supplemented with blank space, excluding decimal point.

No.15，16（ASCII）：Space

No.17－22（ASCII）：Price data, the insufficient part shall be supplemented with blank space, excluding decimal point.

For example：

If net weight 2.365；unit price 5.62；price 13.29，send data as follow：

0A0D2032333635202020203536322020202031333239

If net weight 0.000；unit price 0.00；price 0.00，send data as follow：

0A0D2020202030202020202020302020202020202030

If net weight be negative, then directly send

0A0D2020202020202020202020202020202020202020

Protocol G（total 7 bytes）

No. 1（hex）：0x0A

No.2（hex）：0x0D

No.3－7（ASCII）：Weight data, the insufficient part is supplemented with space, excluding the decimal point

For example：

If net weight 2.365，send data as follow：

0A0D2032333635

If net weight -2.365，send data as follow：

0A0D2D32333635

Protocol H（tota 11 bytes）Reply transmission

The upper computer sends the ASCII letter P (capital), and the lower computer responds

If the weight of the scale is negative or unstable

send（hex）	3F 3F 3F 3F 3F total 5 bytes

If the weight of the scale is positive and the weight is stable, send in the following format

No.1（hex）：0x02

No. 2－8（ASCII）：Weight data, the insufficient part is supplemented with space, including the decimal point

No.9 Space

No.10，11 weight unit  for example:KG

If net weight 2.365，send data as follow：

02 20 20 32 2E 33 36 35 20 4B 47